# ANNEX II + III:	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

# Contract title: Procurement of Equipment for Fire and Rescue Units in the Republic of Srpska within the INEREP Project

# Lot 1: Supply of Fire Pick-up Vehicles with 4x4 Drive and accompanying equipment

Publication reference: 25.02-810.2-74/26

Columns 1-2 should be completed by the contracting authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the contracting authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words ‘compliant’ or ‘yes’ are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

The subject of Lot 1 is the supply and delivery of 8 new and unused fire pick-up vehicles with 4x4 drive, superstructure and accompanying fire-fighting and rescue

equipment for fire and rescue units in the Republic of Srpska within the INEREP Project.

The vehicles shall be used for fire-fighting and rescue interventions, including interventions in rural and hard-to-reach areas, field operations, fire suppression, initial

response, transport of fire-fighting equipment and support to civil protection activities.

1. General requirements

1.1 The tenderer shall ensure that the functions and features of the offered vehicles, superstructure and equipment meet all minimum requirements set out in these

Technical Specifications.

1.2 All specification details listed in these Technical Specifications represent minimum requirements, unless stated otherwise. Any improvements or additional

features offered by the tenderer shall be clearly identified in the technical offer.

1.3 The offered vehicles, superstructure and equipment must be new, unused and fully compliant with the required technical specifications and applicable standards.

1.4 The supplies shall include the base vehicle, superstructure, water and foam tanks, high-pressure pump, electrical, signalling and communication equipment,

accompanying fire-fighting and rescue equipment, all required technical documentation, catalogues or technical sheets, certificates, warranty documents, user

manuals, service documentation, maintenance plan and all other documents required for proper delivery, registration, use, maintenance and acceptance.

1.5 The tenderer shall clearly indicate the offered make, model, type and configuration of the vehicle, superstructure and equipment.

1.6 The tenderer shall submit catalogues or technical sheets issued by the manufacturer or authorised distributor, clearly showing the offered model and all requested

technical characteristics.

1.7 The documentation supplied shall clearly indicate, highlight or mark the models offered and the options included, if any, so that the evaluation committee can

identify the exact configuration offered.

1.8 Offers that do not permit the precise identification of the offered models, configuration and technical characteristics may be rejected by the evaluation

committee.

1.9 Where these Technical Specifications refer to a particular standard, make, source, process, trademark, patent, type or origin, the words “or equivalent” shall be

deemed to apply. Any equivalent solution must meet or exceed the required functional, technical, safety and performance requirements. The tenderer shall provide

documentary evidence demonstrating equivalence. This provision shall not permit any reduction of the required quantities, minimum performance levels, capacities,

dimensions or other expressly stated mandatory requirements.

2. Service and maintenance requirements

2.1 The tenderer must submit a certified statement confirming that authorised service for the offered vehicle and equipment exists in Bosnia and Herzegovina.

2.2 The statement must include the name and address of the authorised service provider or providers for the offered vehicle and equipment.

2.3 The tenderer must submit a regular maintenance plan for the offered vehicle and equipment, issued by the manufacturer or authorised service provider of the

offered vehicle and equipment.

3. Warranty conditions

3.1 Warranty shall apply in accordance with Article 32 of the General Conditions, the Special Conditions and these Technical Specifications.

3.2 Warranty for the complete vehicle shall be minimum 5 years or 100,000 km, whichever occurs first from the date of provisional acceptance

3.3 The contractor shall be responsible for maintenance and repair of the supplies within the warranty period, in accordance with the warranty conditions and the

contract.

3.4 The contractor shall guarantee the proper operation of the supplied vehicles, superstructure and equipment. In the event of failure during the warranty period, the

contractor shall restore proper operation or, where repair is not possible, replace the defective item with equivalent supplies at its own cost.

4. Delivery and registration documentation

4.1 For Lot 1, the maximum period of implementation shall be 280 calendar days, starting from the date on which the contract is signed by the last party. The delivery

period shall be 250 calendar days. The vehicles shall be delivered successively in four batches of two vehicles. The first batch shall be delivered no later than two

calendar months after the commencement date, and each subsequent batch shall be delivered at intervals not exceeding two calendar months. All eight vehicles shall

in any event be delivered no later than 250 calendar days after the commencement date.

The period of implementation shall end on the date of issuance of the Provisional Acceptance Certificate. The certificate shall be issued or refused within 30 calendar

days of receipt of the contractor’s complete application for provisional acceptance.

4.2 The delivery location:  The supplies shall be delivered DDP to the Civil Protection Administration of the Republic of Srpska, Spasovdanska 3, 71123 East Sarajevo,

Republic of Srpska, Bosnia and Herzegovina.

4.3 Upon delivery of each complete vehicle, including the superstructure and installed equipment, the contractor must provide all documentation necessary for

registration, legal operation, use and provisional acceptance of that vehicle in Bosnia and Herzegovina.

4.4 The documentation required for registration must include the invoice, homologation document, documentation relating to the superstructure and installed

equipment, and the customs declaration, where applicable. All documents must correspond to the make, model, type, chassis number and final configuration of the

delivered vehicle.

4.5 A customs declaration shall be provided where applicable under the customs rules governing the importation of the supplied vehicle.

| 1. Item number | 2. Specifications required | 3. Specifications offered | 4. Notes, remarks, ref to documentation | 5. Evaluation committee’s notes |
| --- | --- | --- | --- | --- |
| 1. | Fire pick-up vehicles with 4x4 drive and accompanying equipment Quantity: 8 vehicles. Each offered vehicle shall comply with the technical requirements listed below. |  |  |  |
| 1.1 | The supplies shall consist of new and unused fire pick- up vehicles with 4x4 drive, double cabin, superstructure and accompanying fire-fighting and rescue equipment, intended for fire-fighting, initial response, field interventions and support to civil protection activities. |  |  |  |
| 1.2 | The vehicle must have a double cabin suitable for operational use by fire and rescue units. |  |  |  |
| 1.3 | Number of seats: 4+1. |  |  |  |
| 1.4 | Drive: 4x4, suitable for use on paved roads, rural roads and field intervention conditions. |  |  |  |
| 1.5 | Engine power: minimum 120 kW. |  |  |  |
| 1.6 | Engine capacity: maximum 2,000 cm³, or equivalent engine solution ensuring the required power, Euro 6 emission standard and operational performance. |  |  |  |
| 1.7 | Emission standard: minimum Euro 6, or equivalent valid emission standard applicable to the offered vehicle. |  |  |  |
| 1.8 | Fuel: diesel. |  |  |  |
| 1.9 | Gearbox: manual or automatic, minimum 6 forward gears + 1 reverse gear, or equivalent transmission solution suitable for the intended operational use. |  |  |  |
| 1.10 | Vehicle length: minimum 5,200 mm, or equivalent dimensions ensuring the required cabin, cargo area and installation of the superstructure and equipment. |  |  |  |
| 1.11 | Wheelbase: minimum 3,100 mm, or equivalent wheelbase ensuring the required operational stability, cargo area and superstructure functionality. |  |  |  |
| 1.12 | Cargo area dimensions: minimum 1,490 x 1,520 x 490 mm, or equivalent dimensions ensuring proper installation of the required superstructure, tanks, pump and accompanying equipment. |  |  |  |
| 1.13 | Wheels: minimum 16 inches, aluminium or equivalent suitable wheels for the intended operational use. |  |  |  |
| 1.14 | The vehicle must be supplied with a full-size spare wheel with steel rim or equivalent suitable rim. |  |  |  |
| 1.15 | The vehicle must be supplied with a jack, wheel replacement set, warning triangle and reflective vest. |  |  |  |
| 1.16 | Colour of the base vehicle: fire red, RAL 3000 or equivalent. |  |  |  |
| 1.17 | Braking system: ABS and ESP, or equivalent safety systems. |  |  |  |
| 1.18 | The vehicle must be equipped with hill start assist and downhill assist systems, or equivalent systems suitable for safe operational use. |  |  |  |
| 1.19 | The vehicle must have front and rear electric windows. |  |  |  |
| 1.20 | The vehicle must be equipped with a lane departure warning system. |  |  |  |
| 1.21 | The vehicle must have central remote locking. |  |  |  |
| 1.22 | The vehicle must be equipped with an automatic emergency braking system. |  |  |  |
| 1.23 | The vehicle must have front airbags for the driver and front passenger. |  |  |  |
| 1.24 | The vehicle must have electrically adjustable and heated mirrors. |  |  |  |
| 1.25 | The vehicle must have radio with USB and Bluetooth function, or equivalent audio/connectivity system. |  |  |  |
| 1.26 | The vehicle must have cruise control. |  |  |  |
| 1.27 | Warranty for the complete vehicle: minimum 5 years or 100,000 km, whichever occurs first. |  |  |  |
| 1.28 | The vehicle must be new and unused. |  |  |  |
| 1.29 | The tenderer must submit a catalogue or technical sheet issued by the manufacturer or authorised distributor, clearly showing all requested technical characteristics of the offered vehicle. |  |  |  |
| 2 | Superstructure of each vehicle |  |  |  |
| 2.1 | The superstructure must be installed inside the cargo area of the vehicle chassis and must be suitable for installation and safe use of the required fire-fighting equipment. |  |  |  |
| 2.2 | The superstructure must be installed on the main chassis and made of lightweight materials, with internal lining made of high-quality plastic or equivalent suitable material resistant to operational use. |  |  |  |
| 2.3 | The colour of the superstructure must be identical or equivalent to the colour of the base vehicle, fire red RAL 3000 or equivalent. |  |  |  |
| 2.4 | The superstructure must be fully closed, with side lifting doors on the left and right sides and two-part rear doors. |  |  |  |
| 2.5 | The two-part rear doors must be designed so that the upper part opens upwards and the lower part opens downwards, enabling full access to the rear part of the superstructure and easier access to fire-fighting equipment. |  |  |  |
| 2.6 | Maximum height of the vehicle with superstructure: 2,000 mm. |  |  |  |
| 2.7 | The tenderer must submit a catalogue or technical sheet issued by the manufacturer or authorised distributor, clearly showing all requested technical characteristics of the offered superstructure. |  |  |  |
| 3 | Water and foam tanks of each vehicle |  |  |  |
| 3.1 | Polypropylene water and foam tanks must be installed inside the superstructure |  |  |  |
| 3.2 | The water tank and the high-pressure pump must be fixed inside the base cargo area by bolts and steel profiles or by an equivalent safe fixing system suitable for vehicle movement and intervention conditions. |  |  |  |
| 3.3 | Water tank capacity: minimum 350 litres. |  |  |  |
| 3.4 | Foam tank capacity: minimum 20 litres. |  |  |  |
| 3.5 | The water and foam tanks must form one integrated unit, with the foam tank integrated inside the water tank. |  |  |  |
| 3.6 | The tanks must be equipped with mechanical level indicators. |  |  |  |
| 3.7 | Tank filling must be possible through a Storz C 52 mm connection or equivalent compatible connection. |  |  |  |
| 3.8 | The filling connection must be equipped with a butterfly valve and closed with a blind coupling. |  |  |  |
| 3.9 | The tank must have an inspection opening with a minimum diameter of 150 mm. |  |  |  |
| 3.10 | Two hoses with a diameter of 3/4 inch must be installed from the tank, with taps for filling backpack sprayers. |  |  |  |
| 3.11 | Water from the hoses for filling backpack sprayers must flow by gravity. |  |  |  |
| 3.12 | The water and foam tanks must be new and unused. |  |  |  |
| 3.13 | The tenderer must submit a catalogue or technical sheet issued by the manufacturer or authorised distributor, as well as a technical drawing and 3D model, clearly showing all requested technical characteristics of the offered tanks and installation. |  |  |  |
| 4 | High-pressure pump of each vehicle |  |  |  |
| 4.1 | The high-pressure pump must be installed in the rear part of the superstructure, behind the water and foam tanks. |  |  |  |
| 4.2 | The pump must be supplied with water from the tank through a reinforced hose with a diameter of 1 inch or equivalent suitable connection. |  |  |  |
| 4.3 | The high-pressure pump must be equipped with a two-cylinder, four-stroke internal combustion petrol engine, with minimum engine capacity of 480 cm³ and minimum engine power of 11.6 kW. |  |  |  |
| 4.4 | The engine must have an integrated petrol fuel tank with minimum capacity of 8 litres. |  |  |  |
| 4.5 | Exhaust gases must be discharged outside the superstructure. |  |  |  |
| 4.6 | The fire-fighting pump must have a minimum flow rate of 36 l/min at a pressure of 100 bar. |  |  |  |
| 4.7 | Pump ignition must be provided by a 12 V electric starter, key-operated, and by a manual emergency starter in case of electric starter failure. |  |  |  |
| 4.8 | The pump must be equipped with a hose with minimum length of 60 m and minimum diameter of 12 mm. |  |  |  |
| 4.9 | The hose must be placed on a reel equipped with brake and locking system, with manual hose rewinding. |  |  |  |
| 4.10 | A nozzle with foam attachment must be installed at the end of the hose. |  |  |  |
| 4.11 | The nozzle must have an adjustable stream, from full flow to water shield/fog. |  |  |  |
| 4.12 | The pump must have an integrated foam mixing system in the range from 0% to 6%. |  |  |  |
| 4.13 | The weight of the pump with full fuel tank must not exceed 160 kg. |  |  |  |
| 4.14 | The pump must be new and unused. |  |  |  |
| 4.15 | The tenderer must submit a catalogue or technical sheet issued by the manufacturer or authorised distributor, clearly showing all requested technical characteristics of the offered high-pressure pump. |  |  |  |
| 5. | Electrical, signalling and communication equipment of each vehicle |  |  |  |
| 5.1 | Each offered vehicle must be equipped with light and sound signalling in accordance with the applicable road traffic legislation in Bosnia and Herzegovina. |  |  |  |
| 5.2 | Two blue LED flashing lights must be installed at the front part of the vehicle, in the vehicle grille. |  |  |  |
| 5.3 | Two blue LED flashing lights must be installed at the rear part of the vehicle or superstructure. |  |  |  |
| 5.4 | A light and sound signalling block must be installed on the roof of the vehicle cabin. |  |  |  |
| 5.5 | The light and sound signalling block must include integrated blue LED flashing lights, siren with megaphone and loudspeaker. |  |  |  |
| 5.6 | The light and sound signalling block must be controlled from inside the vehicle cabin. |  |  |  |
| 5.7 | A VHF radio station must be installed inside the vehicle cabin. |  |  |  |
| 5.8 | The VHF radio station must have minimum 150 channels. |  |  |  |
| 5.9 | The frequency range of the VHF radio station must be between 136 MHz and 174 MHz. |  |  |  |
| 5.10 | The weight of the VHF radio station must not exceed 1.4 kg. |  |  |  |
| 5.11 | An electric self-recovery winch must be installed at the front part of the vehicle. |  |  |  |
| 5.12 | The electric winch must have minimum pulling capacity of 5,400 kg. |  |  |  |
| 5.13 | The electric winch must have minimum motor power of 6.8 HP. |  |  |  |
| 5.14 | Cable length of the electric winch: minimum 28 m. |  |  |  |
| 5.15 | The weight of the electric winch must not exceed 59 kg. |  |  |  |
| 5.16 | The electrical, signalling and communication equipment must be new and unused. |  |  |  |
| 5.17 | The tenderer must submit catalogues or technical sheets issued by the manufacturer or authorised distributor for the LED flashing lights, light and sound signalling block, VHF radio station and electric self-recovery winch. |  |  |  |
| 6. | Accompanying equipment inside the superstructure of each vehicle |  |  |  |
| 6.1 | Each vehicle superstructure must be equipped with holders and storage arrangements for the accompanying equipment listed below. The quantities of accompanying equipment listed under this section shall be provided per vehicle. |  |  |  |
| 6.2 | Chainsaw: minimum power 2.7 HP, minimum engine capacity 40 cm³, maximum weight 4.7 kg - 1 piece. |  |  |  |
| 6.3 | Fire-fighting backpack sprayers: minimum capacity 25 litres - 10 pieces. |  |  |  |
| 6.4 | Fire-fighting axe with wooden handle: minimum handle length 900 mm, maximum weight 2.9 kg - 3 pieces. |  |  |  |
| 6.5 | Pickaxe with wooden handle: minimum handle length 700 mm, maximum weight 2.8 kg - 3 pieces. |  |  |  |
| 6.6 | Shovel with wooden handle: minimum handle length 1,500 mm, maximum weight 2.5 kg - 3 pieces. |  |  |  |
| 6.7 | Fire beater with wooden handle: minimum handle length 2,000 mm, minimum head width 500 mm, maximum weight 3.0 kg - 3 pieces. |  |  |  |
| 6.8 | The accompanying equipment must be new and unused. |  |  |  |
| 6.9 | The tenderer must submit a catalogue or technical sheet for each item of accompanying equipment, issued by the manufacturer or authorised distributor, clearly showing the required characteristics. |  |  |  |